Baseline Operational Risk Assessment

Skyline Roof Surveys (Mavic 3)  ·  Skyline Roof Surveys Ltd

Generated 2026-07-10  ·  Template 0.2.0

DRAFT — accountable-manager review and adoption required before operational use.

| Website prefilled | Baseline hazards, controls and the named remote pilot as the default control owner. |
| --- | --- |
| Operator action | Confirm final ratings after the site survey, add site hazards, record the decision and sign before flight. |

This baseline is tailored to the selected launch and recovery methods. Before each operation, the remote pilot confirms the controls, assigns a final residual rating and adds site-specific hazards on the site survey.

| UAS operator / authorisation | Skyline Roof Surveys Ltd / [SAMPLE] PDRA01-123456 |
| --- | --- |
| Aircraft | DJI Mavic 3 Enterprise / 1581F5FHD228Q00A17XX |
| Remote pilot | J. Smith |
| Launch / recovery | Vertical take-off / Vertical landing |
| Assessment owner / review date | J. Smith / 2026-07-10 |

## Risk acceptance method

- Low — controls are established and the activity may proceed if site conditions match.

- Medium — remote pilot confirms the listed controls and records the decision before flight.

- High — do not proceed until additional controls reduce the risk and the accountable manager accepts the revision.

## Baseline hazard register

| Hazard and cause | Potential consequence | Required controls | Residual / owner |
| --- | --- | --- | --- |
| Loss of control Cause: C2 link, flight-controller or pilot error | Impact with people, property or another aircraft | Pre-flight function checks; tested lost-link response; emergency landing area; observer where used | Baseline: Medium Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |
| Flyaway Cause: GNSS/compass error or unresolved link loss | Uncontrolled flight or airspace infringement | GNSS/compass status check; flight-volume limit where fitted; flyaway and notification procedure | Baseline: Medium Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |
| Battery or fuel fire Cause: Damage, over-discharge, charging fault or leak | Fire, burns or property damage | Condition checks; manufacturer-compatible charging/storage; isolation equipment; fire emergency procedure | Baseline: Low Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |
| Propeller / propulsion injury Cause: Arming or handling near moving components | Cuts or impact injury to crew | Arming discipline; clear launch area; crew brief; guards/PPE where appropriate | Baseline: Low Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |
| Collision with people Cause: Control loss or ground incursion | Injury | A minimum horizontal separation of 50 m must be maintained from uninvolved persons, reducible to 30 m during take-off and landing. Assemblies of people must never be overflown, and a minimum horizontal separation of 50 m from assemblies must be kept (also observing the 1:1 rule: horizontal separation at least equal to the height of the aircraft). | Baseline: Medium Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |
| Collision with property / obstacles Cause: Survey omission, control error or poor positioning | Damage and secondary injury | Onsite survey; obstacle clearance; defined operating and contingency volumes | Baseline: Low Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |
| Adverse weather Cause: Wind, precipitation, poor visibility or temperature | Control degradation or loss of VLOS | Manufacturer and manual limits; aviation forecast; onsite re-check; explicit abort criteria | Baseline: Low Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |
| GNSS degradation Cause: Interference, obstruction or space weather | Loss of position hold or navigation assistance | GNSS status check; competent non-GNSS control; increase separation and land | Baseline: Medium Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |
| Airspace conflict Cause: Manned-aircraft incursion or incorrect airspace information | Collision risk / airprox | Current airspace, FRZ and NOTAM checks; visual scan; descend/land; Airprox process | Baseline: Medium Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |
| Public entering the operating area Cause: Uncontrolled or unmonitored access point | Injury risk and loss of separation | Cordon; access control; marshal where required; hold or land on incursion | Baseline: Low Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |
| Data / privacy incident Cause: Unnecessary capture, loss or unauthorised disclosure | Privacy harm or regulatory breach | Data and privacy procedure; minimisation, access control and retention limit | Baseline: Low Final: OPERATOR ENTRY REQUIRED — confirm after site survey Owner: J. Smith |

## Approval

| Additional hazards recorded on site survey | YES / NO |
| --- | --- |
| Overall residual risk | LOW / MEDIUM / HIGH |
| Decision / signatures | Remote pilot J. Smith: OPERATOR ENTRY REQUIRED — sign and date Accountable manager J. Smith if required: OPERATOR ENTRY REQUIRED — sign and date |