Pre-Flight Checklist & Briefing Record

Skyline Roof Surveys (Mavic 3)  ·  Skyline Roof Surveys Ltd

Generated 2026-07-10  ·  Template 0.2.0

DRAFT — accountable-manager review and adoption required before operational use.

| Website prefilled | Flight identity, declared aircraft checks, routine crew, communication, limits and emergency arrangements. |
| --- | --- |
| Operator action | Complete site-specific briefing details, tick the release checks and obtain attendee sign-on before flight. |

Complete and retain for each operating session; this document also provides the CAP 2606 Annex 3 briefing record.

## Flight identity

| Date / site / flight-log entry | OPERATOR ENTRY REQUIRED |
| --- | --- |
| UAS operator / Operator ID | Skyline Roof Surveys Ltd / [SAMPLE] OP-XXXXXXXXXXXX |
| PDRA01 authorisation | [SAMPLE] PDRA01-123456 |
| Remote pilot / Flyer ID | J. Smith / [SAMPLE] FLY-XXXXXXXXXXXX |
| Aircraft / serial | DJI Mavic 3 Enterprise / 1581F5FHD228Q00A17XX |

## Aircraft and operational release

| ✓ | Check / action | Evidence / initials |
| --- | --- | --- |
| ☐ | Feasibility and onsite survey complete; GO decision recorded |  |
| ☐ | Current airspace, FRZ, restriction and NOTAM checks complete |  |
| ☐ | Landowner / airspace permissions or notifications confirmed |  |
| ☐ | Weather within aircraft and Operations Manual limits |  |
| ☐ | Cordon, operating volume, contingency volume and access control established |  |
| ☐ | Primary and emergency landing areas clear and identified |  |
| ☐ | Airframe inspected with no unresolved damage or defect |  |
| ☐ | Propulsion components secure and serviceable |  |
| ☐ | Battery / fuel identified, serviceable and sufficient; landing reserve planned |  |
| ☐ | Payload secure; actual take-off mass within MTOM; centre of gravity acceptable |  |
| ☐ | CAA Operator ID displayed on the airframe |  |
| ☐ | Command unit charged; firmware/configuration correct; C2 link check satisfactory |  |
| ☐ | GNSS, compass and navigation status acceptable where applicable |  |
| ☐ | Flight-volume / geofence limits set to the site |  |
| ☐ | Home point and RTH altitude confirmed above relevant obstacles |  |
| ☐ | Lost-link action confirmed against the tested configuration |  |
| ☐ | No unresolved maintenance item prevents release to service |  |

## Annex 3 — pre-flight briefing record

| Task, flight plan and planned altitude | Default purpose: Survey / inspection; declared maximum planned altitude: 120 ft; OPERATOR ENTRY REQUIRED — add the actual flight plan |
| --- | --- |
| Crew names, roles and positions | Remote pilot: J. Smith; support: A. Taylor — visual observer; OPERATOR ENTRY REQUIRED — confirm actual positions |
| Communication method and standard calls | Direct voice using closed-loop calls; STOP means immediate safe action; radio backup where noise requires it |
| Cordon / public access responsibilities | OPERATOR ENTRY REQUIRED |
| Airspace threats and lookout responsibilities | OPERATOR ENTRY REQUIRED |
| Emergency, lost-link and pilot-incapacitation actions | Lost link: Return home; use Document 08; OPERATOR ENTRY REQUIRED — confirm site-specific actions |
| Emergency contact and assembly point | Skyline duty manager — 07700 900123; OPERATOR ENTRY REQUIRED — add the assembly point |
| Questions / changes from the plan | OPERATOR ENTRY REQUIRED |
| Remote-pilot release decision | GO / NO GO |

### Briefed personnel sign-on

| Name | Role | Briefing understood | Signature / initials | Time |
| --- | --- | --- | --- | --- |
| J. Smith | Remote pilot | YES / NO | OPERATOR ENTRY REQUIRED | OPERATOR ENTRY REQUIRED |
| A. Taylor — visual observer | Support personnel | YES / NO | OPERATOR ENTRY REQUIRED | OPERATOR ENTRY REQUIRED |
| OPERATOR ENTRY REQUIRED | Additional attendee if used | YES / NO | OPERATOR ENTRY REQUIRED | OPERATOR ENTRY REQUIRED |
| OPERATOR ENTRY REQUIRED | Additional attendee if used | YES / NO | OPERATOR ENTRY REQUIRED | OPERATOR ENTRY REQUIRED |